Class 2: Working with Geographic Data

Latitude and longitude are geographic coordinates. These are different from cartesian coordinates (x,y). They represent angles rather than distances.

- Latitude describes the angle made from the equator to a location in the North/South direction. The term meridional describes a direction along a circle of latitude.

- Longitude describes the angle made from the prime meridian to a location in the East/West direction. The term zonal describes a direction along a circle of longitude.

- Latitude is defined to be 0° at the equator. Latitude reaches a maximum of 90° at the North Pole and a minimum of -90° at the South Pole.

  - Latitude can be any value -90°≤θ≤90°.

- Longitude is defined to be 0° at the prime meridian. Longitude is measured as a positive value East of the prime meridian (reaching a maximum of 180° at the anti-meridian) and is measured as a negative value West of the prime meridian (ranging towards a minimum of -180° at the anti-meridian).

  - Longitude can be any value -180°<θ≤180°.

The Western and Southern directions can either be indicated by negative signs, or simply by stating the longitude or latitude are measured W or N, respectively.

- Example: the coordinates of Oregon State University are: 44.5638° N, 123.2794° W

- Alternatively, this could be written as (44.5638° , -123.2794°)

To use geographic coordinates to find distances between two locations we need to perform some trigonometry. We will assume the earth is a sphere (this is in fact an approximation of the actual shape of the Earth, which is better approximated as an oblate spheroid, being flatter at the poles than the equator).

To find the distance, d, between two locations A and B on the Earth, we need to find the distances m and z shown in the diagram below.

The meridional distance, m, is represented by a change in latitude. We can calculate the angular change between locations A and B, ∆latAB, to calculate m.

Look at the diagram above. Maybe you can see how we can calculate the meridional distance m. Is there a way to make right triangles from the isosceles triangle? Can you use trigonometry to find m given you know r (the radius of the Earth) and ∆latAB?

To find m we split the triangle made by ∆latAB in two, forming two right angle triangles. Each of these has hypotenuse r and opposite length m2. Hence we can find m as

m2= rsin∆latAB2

↓

m=2 rsin∆latAB2

Finding the zonal distance, z, is a little more involved because zonal bands decrease in radius as they approach the pole. The figure below illustrates this. A zonal band bounded by two fixed longitudes is outlined in black.   The distance between the two longitudes bounding the zonal band decreases as latitude increases due to the shape of the Earth.

The zonal band as seen from looking down on the North Pole

In the diagram above, see that we need to find the radius of the longitudinal band, l,  before calculating z. This radius, l, is the same as r at the equator and decreases to 0 at the north pole. You can find l if you know the latitude of the zonal band.

Using the right triangle in the above diagram, we can see that

l=rcos(latB)

Splitting the isosceles triangle in two as we did on the previous page, we can solve for z as

z2=lsin∆lonAB2

z= 2 lsin∆lonAB2

Finally, combining the equations for l and z, we can solve for z as

z= 2 rcos(latB)sin∆lonAB2

Now we know m and z we can find distance d and bearing C exactly as we did in cartesian coordinates.